- Notes/Revisions on the LM384 Amplifier Project

- LED

  - Mismatch between the footprint used for the LED and the physical component used for assembling the final project.

  - Component used part number: HV-31210/260/SUR-U1930

- Potentiometer

  - Nonconventional orientation for volume adjustment such that the counterclockwise direction functions as volume-up while the clockwise direction functions as volume-down.

  - Component used part number: PTV09A-2020F-A103

- Ferrite Bead

  - The band where resistive region is significant falls in the tens of megahertz, which is well above the expected power supply noise. A possible solution is to consider reselecting it or replacing it with an inductor.

  - Component used part number: 28C0236-0EW-10

- Routing

  - Placement of a trace right underneath the heatsink used for the LM317, which might result in an unwanted connection.

    - Referred trace net: C2-Pad1

    - Referred heatsink part number: HSE-B2111-038

  - Placement of a trace right underneath the heatsink used for the LM384, which might result in an unwanted connection.

    - Referred trace net: VS

    - Referred heatsink part number: 580200B00000G

All part numbers used are the manufacturer part number.